三层交换机命令

<Huawei>sys

Enter system view, return user view with Ctrl+Z.

[Huawei]undo info-center enable

Info: Information center is disabled.

[Huawei]sysname SW2

[SW2]

[SW2]interface g0/0/1

[SW2-GigabitEthernet0/0/1]port link-type trunk

[SW2-GigabitEthernet0/0/1]port trunk allow-pass vlan all

[SW2-GigabitEthernet0/0/1]

[SW2-GigabitEthernet0/0/1]display this

#

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 2 to 4094

#

return

[SW2-GigabitEthernet0/0/1]q

[SW2]

[SW2]

[SW2]vlan batch 10 20 30

Info: This operation may take a few seconds. Please wait for a moment...done.

[SW2]

[SW2]interface vlanif10

[SW2-Vlanif10]ip address 192.168.10.1 24

[SW2-Vlanif10]q

[SW2]interface vlanif20

[SW2-Vlanif20]ip address 192.168.20.1 24

[SW2-Vlanif20]q

[SW2]interface vlanif30

[SW2-Vlanif30]ip address 192.168.30.1 24

[SW2-Vlanif30]q

[SW2]

实验验证（见下图）