二层交换机命令

<Huawei>sys

Enter system view, return user view with Ctrl+Z.

[Huawei]undo info-center enable

Info: Information center is disabled.

[Huawei]sysname SW1

[SW1]

[SW1]vlan batch 10 20 30

Info: This operation may take a few seconds. Please wait for a moment...done.

[SW1]

[SW1]interface e0/0/1

[SW1-Ethernet0/0/1]port link-type access

[SW1-Ethernet0/0/1]port default vlan 10

[SW1-Ethernet0/0/1]q

[SW1]interface e0/0/2

[SW1-Ethernet0/0/2]port link-type access

[SW1-Ethernet0/0/2]port default vlan 20

[SW1-Ethernet0/0/2]q

[SW1]interface e0/0/3

[SW1-Ethernet0/0/3]port link-type access

[SW1-Ethernet0/0/3]port default vlan 30

[SW1-Ethernet0/0/3]q

[SW1]

[SW1]interface e0/0/4

[SW1-Ethernet0/0/4]port link-type trunk

[SW1-Ethernet0/0/4]port trunk allow-pass vlan 10 20 30

[SW1-Ethernet0/0/4]