<Huawei>sys

Enter system view, return user view with Ctrl+Z.

[Huawei]undo info-center enable

Info: Information center is disabled.

[Huawei]sysname server

[server]interface g0/0/0

[server-GigabitEthernet0/0/0]ip address 10.1.1.2 24

[server-GigabitEthernet0/0/0]q

[server]

[server]user-interface vty 0 2

[server-ui-vty0-2]

[server-ui-vty0-2]authentication-mode aaa

[server-ui-vty0-2]

[server-ui-vty0-2]protocol inbound ?

all     All protocols

ssh     SSH protocol

telnet  Telnet protocol

[server-ui-vty0-2]protocol inbound ssh

[server-ui-vty0-2]

[server-ui-vty0-2]q

[server]

[server]

[server]

[server]aaa

[server-aaa]local-user szxy password cipher nc@3601  privilege level 15

Info: Add a new user.

[server-aaa]

[server-aaa]local-user szxy service-type ssh

[server-aaa]

[server-aaa]q

[server]

[server]

[server]ssh user szxy authentication-type password

Authentication type setted, and will be in effect next time

[server]

[server]

[server]stelnet server enable

Info: Succeeded in starting the STELNET server.

[server]

[server]rsa local-key-pair create

The key name will be: Host

% RSA keys defined for Host already exist.

Confirm to replace them? (y/n)[n]:y

The range of public key size is (512 ~ 2048).

NOTES: If the key modulus is greater than 512,

It will take a few minutes.

Input the bits in the modulus[default = 512]:512

Generating keys...

...........++++++++++++

...............++++++++++++

.......++++++++

............++++++++

[server]

[server]

[server]

[server]display ssh serve status

SSH version                         :1.99

SSH connection timeout              :60 seconds

SSH server key generating interval  :0 hours

SSH Authentication retries          :3 times

SFTP Server                         :Disable

Stelnet server                      :Enable

[server]